Colorizing Mars

Documented proposed solution or strategy

This section documents the strategy for coloring high-resolution black-and-white Mars imagery using low-resolution color data. The approach combines high-performance geospatial libraries (Rasterio) with computer vision libraries (OpenCV, scikit-image) to handle alignment, color separation, and filtering. The core idea is to use the L*a*b* color space to separate luminance (detail) from chrominance (color) and apply histogram matching for color consistency.

Approach Details:

- Read Input Files:

  - Read the high-resolution B&W image (JPEG or GeoTIFF) using cv2.imread for simple images or rasterio for GeoTIFFs.

  - Read the low-resolution color GeoTIFF using rasterio.

- Align Data:

  - The low-resolution image data, read as a NumPy array.

  - Convert the low-resolution image data from (Channel, Height, Width) to (Height, Width, Channel) to match the high-resolution image.

  - Resize the low-res image to the high-res dimensions using cv2.resize with interpolation (INTER_CUBIC or INTER_LINEAR).

  - This step allows both images to be processed pixel-to-pixel.

- Convert to L*a*b*:

  - Convert the resized low-res RGB data to the L*a*b* color space using cv2.cvtColor(..., cv2.COLOR_RGB2LAB).

  - LAB separates brightness (L*) from color channels (a*, b*), making it easier to manipulate color without affecting details.

    - L* → Luminance (lightness / brightness / detail)

    - a* → Chrominance channel (green → red)

    - b* → Chrominance channel (blue → yellow)

- Histogram Matching:

  - Use skimage.exposure.match_histograms on the luminance channel.

  - This updates the pixel intensities of the high-res image to match the statistical distribution of the low-res luminance channel.

  - Ensures brightness in the high-res image follows the low-res reference.

Syntax: skimage.exposure.match_histograms(image, reference, *, channel_axis=None, multichannel=False)

- Refine Chroma Channels:

  - Smooth or sharpen the a* and b* channels using edge-preserving filters:

- cv2.bilateralFilter: Smooths colors based on proximity and color similarity.

- cv2.ximgproc.guidedFilter: Uses the high-res luminance channel as a guide to preserve sharp edges in the color data.

- Reconstruct

  - Combine the matched L* channel with the filtered a* and b* channels into a final L*a*b* array.

  - Convert this back to RGB (cv2.cvtColor(..., cv2.COLOR_Lab2RGB)).

- Georeference & Save:

- Use rasterio to save the output image as a GeoTIFF ,by using the low-res GeoTIFF metadata, bounds, and CRS to save the high-resolution colorized image as a GeoTIFF using Rasterio.

- This preserves spatial referencing for GIS applications or Cesium.

Concluding Note:
To my knowledge, in previous projects I have used OpenCV and CNNs (Neural Networks) for image resolution enhancement. For these Mars images, I experimented with OpenCV and Rasterio. The challenge was that one image was JPEG and the other a GeoTIFF. I initially used OpenCV combined with Rasterio to maintain georeferencing, and through research and experimentation, I learned how to use OpenCV Lab* color space to separate brightness and color, histogram matching for reference-based adjustments, and bilateral and guided filters for edge-preserving color refinement.Since OpenCV is implemented in C++, this entire workflow can also be performed in C++ for maximum performance.

output.tif

cesium_mars_resolution.ipynb

bentley_cesium